Quick Start Guide

Quick Start Guide

GoVision

Introduction

This quick guide manual describes the features and functions of the HIMS GoVision transportable electronic magnifier. Please read this manual thoroughly before using your GoVision, and keep it in a safe place for easy access should you need to refer to it at a later time.

This User’s guide also includes product warranty information and safety precautions. If there are any updates to the GoVision specifications or features, this manual will be updated to reflect these changes and placed on the GoVision section of the www.hims-inc.com website.

Copyright

GoVision products and intellectual property, including documentation copyrights, are protected by copyright laws, with copyright belonging to JAWON Medical CO., LTD. and HIMS Inc., and may not be used without expressed permission from JAWON Medical CO., LTD. This User Guide may not be copied in whole or part, without permission.

JAWON Medical is the developer and manufacturer of all HIMS brand products for the blind and visually impaired. JAWON Medical is the parent company of HIMS Inc., a wholly-owned subsidiary based in Austin, TX.

Copyright ⓒ 2016 JAWON MEDICAL CO., LTD. 
All rights reserved.

Safety Precautions

To ensure your personal safety and the proper use of the product as well as to prevent accidental damage, please be aware of and abide by the following safety precautions.

WARNING! For Your Safety

Failing to follow the below precautions may cause serious injury or death.

Do not attempt to disassemble or repair the product yourself. Only authorized repair technicians should disassemble or repair the GoVision, Unauthorized disassembly or repair of the GoVision will void the warranty of the product.

Handle the GoVision with care, as it contains delicate parts. Damage caused by dropping, or external force or shock is not covered by the warranty.

Please keep the GoVision away from extreme temperatures or humid conditions. Do not use the GoVision in environments above 104 degrees Fahrenheit (40 degrees Celsius) or lower than minus 50 degrees Fahrenheit (10 degrees Celsius).

Do not connect the GoVision to ANY electronic devices not indicated in this manual.

Do not use excessive force when adjusting the camera and camera arm.

Install the product on a flat, level surface.

Keep GoVision and its accessories away from infants, small children or pets.

Safety Precautions

Failing to follow the precautions marked with this sign, may result in personal injury or property damage.

Do not use loose power outlets, or a damaged power connector or cord.

Do not touch the product, the power connectors, or a wall outlet with wet hands.

Do not plug multiple power devices into the same outlet as GoVision.

Do not bend or pull the cord to move the unit.

Use only the power supply provided with the product by the manufacturer, or a power supply meeting the electrical specifications provided by the company.

Please keep GoVision away from hazardous or flammable substances.

Always consult our service personnel or an electrician before using an adapter that did not come with the product.

Product Information

Package Contents

Please ensure all of the following items are included in your packing box. If any items are missing, please contact the dealer from whom you purchased the product.

Note

Content may differ slightly from the actual components, as items are subject to change without prior notice to improve the quality of the product.

| Number | Name |
| --- | --- |
|  | GoVision |
|  | Remote control |
|  | Power adapter |
|  | HDMI cable |
|  | Headphone |
|  | Quick start guide |
|  | USB memory |
|  | Optional X/Y moving table |

Physical Orientation

Please take a moment to familiarize yourself with the various parts and controls of the GoVision.

- Front Face

| Number | Name |
| --- | --- |
|  | Jog Switch |
|  | Near View Mode Lens |
|  | Monitor |
|  | Power Switch |
|  | Camera Arm |
|  | Camera Connector |
|  | Camera |

- Back Face

| Number | Name |
| --- | --- |
| ① | Remote Control Holder |
| ② | Lock Lever |
| ③ | Remote Control Port |
| ④ | Power Port |
| ⑤ | HDMI Port |
| ⑥ | USB Port |

Set up GoVision

Powering Up

Connect the power adapter to the power port on the back of the product.

Connect the power cord to a wall outlet.

Remote Control Setup

Connect the end of the remote control's cable to the Remote Port on the back of the GoVision. The remote control is ready to use.

Connecting Headphones

It may be advantageous to use earphones in noisy environments or in environments where you do not wish to disturb those around you.

Connect the earphone to the headphone jack located on the right side of the top of the remote control.

Basic Functions

Power ON / OFF

- Press and hold the Power button for more than 3 seconds.

The standby screen appears, then the unit powers on.

To turn the power off, press the Power button again for more than 3 seconds.

Camera Setup

- Place the lock lever in the horizontal position.

Pull up on the camera arm to adjust the height.

Return the lock lever back to the vertical position.

Adjust the location of the camera by rotating the camera arm.

You may move the camera left, right, up or down.

Using Near View Mode

Use Near View to read or view things at close range, such assigning a check or viewing a photo.

- Rotate the camera arm so that the camera faces the front of the product.

Make sure the camera is pointed downward.

Ensure the Near View lens is closed.

Using Distance View Mode

Use when viewing distant objects, such as a blackboard in a classroom.

- Rotate the camera arm so that it faces the back of the product.

Point the camera so that it is facing the distant target.

Open the Near View lens.

Using Self View Mode

Use as a mirror with the added benefit of magnification when applying makeup or performing other facial grooming.

Note

The maximum distance for Self View Mode is 45 cm from the camera. Keep in mind that images further than that distance will likely appear out of focus.

- Rotate the camera so that it is facing the front of the product.

Point the camera toward your face.

Set LED

Note

This setting applies only in in Near View Mode. LED Lights turn off automatically when switching to Distance View Mode or Self View mode.

LED ON/OFF

- Press and hold the center of the Jog Switch for more than 2 seconds.

- Each time you press this command, it toggles the LED on and off.

Using the Remote Control

| Order | Button | Function |
| --- | --- | --- |
| ① |  | Increase and Decrease Magnification |
| ② |  | Adjust Color Mode |
| ③ |  | Contrast Adjustment |
| ④ |  | Move Up, Down, Left, and Right |
| ⑤ |  | Select |
| ⑥ |  | FREEZE |
| ⑦ |  | Execute OCR (optical character recognition) |
| ⑧ |  | File Browser |
| ⑨ |  | Menu, to set User Preferences |
| ⑩ |  | Set Marker |

Advanced Functions

User Preferences

You can use the menu to customize many of the features of GoVision.

To open the menu, press [] on the remote control.

Selecting Options

| Button | Functions |
| --- | --- |
|  | - Activate the current menu item - Toggle a check box |
| or | - Enter or Exit menu - Adjust the value of the current setting |
| or | - Move among menu and list items |

Menu Items

| Order | Items |  |
| --- | --- | --- |
| 1 | Light |  |
| 2 | Focus Lock |  |
| 3 | OCR Language |  |
| 4 | Digital Photo Frame |  |
| 5 | Digital Clock |  |
| 6 | Advanced options | Color Mode Settings |
|  |  | Marker Settings |
|  |  | Voice Settings |
|  |  | OCR Settings |
|  |  | Language Settings |
|  |  | Digital Photo Frame Settings |
|  |  | Digital Clock Settings |
|  |  | Screen Saver settings |
|  |  | Camera Frequency |
|  |  | My Settings |
|  |  | Initialization |
|  |  | Product Information |

Screen Magnification

Increase and Decrease Magnification

You can adjust the screen magnification in 68 increments.

- Press [] to increase magnification, and [] decrease it.

The current level is displayed on the screen.

Check current Reading Position

- Press and hold [] simultaneously.

The document is reduced to normal size, and your current position is marked with a cross shape [ + ].

Release the Zoom buttons and you are returned to your previous position and magnification level.

Color Mode

Note

In addition to changing the general color modes, you can use the menu to define 4 of the color modes with text and background colors of your choosing.

Cycle through the enabled color modes.

To change the Color Mode on the current screen, press []

Contrast Control

Due to variations in paper type, text type and color and the background color of documents or books that you are reading, you may need to adjust the contrast from 1 to 40 steps to read the documents or books clearly.

- Press [] on the remote control.

- To increase contrast press []. To decrease contrast press [].

To save the contrast setting, press [].

Note

To adjust the contrast, press and hold [] while pressing [] or [] on the remote control

Markers

Markers help you to focus on the text that you are reading by narrowing the display area with window markers or by underlining the sentence that you are reading.

Use [] on the remote control to cycle through the available markers: [Horizontal Line Marker], [Vertical Line Marker], [Horizontal Window Marker] and [Vertical Window Marker].

Screen Capture

To display a still image for a closer look at the content, or to save an image for later viewing, you can capture an image and save the content to a USB flash drive.

Screen Capture

- Press the [], and the current image is frozen on the screen [] and a camera snapshot sound is heard.

When the image is frozen, you may adjust magnification or color modes, just as with the live view.

- To return to live viewing, press [] again.

Saving a Still Image

- Connect the USB cable to the USB port on the back of the product.

When the image is frozen, press and hold [] for more than 2 seconds.

When “Save photo done” [] is displayed and spoken, the image is saved to the USB drive.

Note

- If the USB drive is not connected correctly, the message "Save file failed" is displayed on the screen. The image is saved to the USB as /GoVision/Image/MMDDYYYYhhmm.png.

OCR (Optical Character Recognition)

OCR allows you to read text material aloud using the built-in text-to-speech.

- When the document is displayed on the monitor, press [].

The text of the document is analyzed and begins reading aloud.

Use [] or [] to read the next and previous sentence, use the [] or [] to move by paragraph.

Press [] while reading and playback is paused, pressing [] again resumes reading.

To exit the OCR function, press []

File Viewer

You can use the File Viewer to view saved images, OCR scans, videos, or text-based files.

Connect the USB

- Connect the USB cable to the USB port on the back of the product.

- Press [] on the remote control.

Image, video, and document files and folders appear on the screen.

Use [] or [] to navigate to your desired file, then press [].

To close the document, press [].

Document View Setting

| Button | Function |
| --- | --- |
|  | Increase and Decrease Magnification |
|  | Change Color Mode |
| Press [] or [] | Navigate by line |
| Press [] or [] twice in quick succession | Navigate by page |

Supported file formats

| Option | Format |
| --- | --- |
| Document | MS Word 97~2014 (.doc, .docx, .dot, .dotx), HWP 97~3.0, 2002~2014 (.hwp), Text (.txt, .asc), Rich Text (.rtf) |
| Image | PNG, BMP, JPG(JPEG) |
| Video(Codec) | H.263, H.264 AVC |
| Subtitle | smi, srt |

Voice Guide

You can adjust speed, volume and pitch of the voice guide as well as turn it on and off.

- Press [] on the remote control and select [Advanced Options].

Select [Voice Attribute Setting], then press [] or [].

Select [Voice Guide], then press [] or [].

Focus Lock

If there is movement in the camera view, such as when you are writing under it, you may wish to lock the focus so that it doesn't try to follow your hand or pen.

- Press [] on the remote control.

Select [Focus Lock], then press [] or [].

The lock symbol [] appears on the screen.

To turn off, Focus Lock, press [] or []. The lock symbol [] disappears.

Add-Ons

Digital Photo Frames

You can view images on a USB drive on GoVision's monitor as a slide show.

Note

The first time you connect a USB drive to the product, a [GoVision] folder is automatically created. If the folder does not exist, disconnect and reconnect the USB drive.

Digital Frame Playback

- Save the images in the [Slide] folder of the [GoVision] folder on the USB drive.

Connect the drive containing the photos to the USB port on the back of the product.

Press [] on the remote control.

Select [Digital Photo Frame], then press [] or [].

Pictures in the [GoVision] folder are sequentially displayed on the monitor.

Note

Press [] and [] simultaneously to start and stop the Digital Photo Frame.

Digital Clock

While using GoVision, you can easily check the time with the full-screen digital clock.

Note

Press [] and [] simultaneously and the Digital Clock appears on the screen.

Digital Clock Display

- Press [] on the remote control.

Select [Digital Clock], then press [] or [].

To return to the previous screen, press [] or [].

Screen Savers

In order to protect the screen of the product, you may use the clock or your digital photos as a screen saver.

- Press [] on the remote control and select [Advanced Options].

- Select [Screen Saver Settings], then press [] or [].

Select [Digital Clock] or [Digital Photo Frame], then press [] or [].

Select [Timer Setting], then press [] or [].

Select [Screen saver operation based on time], then press [] or [].

Note

To activate the screen saver, press [], [], [] simultaneously. If you press any button on the camera or remote control, or if the camera detects movement while the screen saver is in operation, the screen saver will be canceled.

Initialization

If something doesn't seem to be operating as you think it should, you can restore all settings to the factory defaults.

| - Press [] on the remote control and select [Advanced Options]. |
| --- |
| Select [Initialization], then press [] or []. |
| Select [Yes], and press []. |
| [Reset done] is displayed on the screen when the rest is complete. |
| Note |  |
| When GoVision is powered off, hold the center of the Jog switch and press and release the Power button. Hold the Jog Switch until the unit reboots, and all settings should be restored to the factory defaults. |  |

GoVision as a Computer Display

You can use the HDMI cable included in the product packaging to connect your GoVision to a PC or Mac and use it as a monitor.

- Connect the HDMI cable to the HDMI port of the GoVision.

Connect the HDMI / DVI adapter to the computer monitor port.

While GoVision is powered on, briefly press and release the Power button.

The computer image is shown on the GoVision screen.

Note

When the GoVision is connected to a computer, the screen saver will not be activated, even if no activity occurs for the period of time you set in the Screen Saver Settings. To change the setting, on the remote control, press [], [], [] simultaneously.

Note

If your computer supports only DVI port, use the HDMI/DVI converter.

Quick Start Button Guide

| No | Function | Remote Control |  | Main Body |
| --- | --- | --- | --- | --- |
| 1 | Power On | - |  | press and hold the Power button for 3 seconds |
| 2 | Power Off | - |  | press and hold The power button for 3 seconds |
| 3 | PC / CCTV Switch | press simultaneously |  | press the power button briefly when powered on |
| 4 | Light On / Off | - |  | press and hold the center position of the Jog Switch |
| 5 | Zoom In/Out | short presses of |  | short presses of the Jog Switch to the left or right |
| 6 | Change Color Mode | short press of |  | short press of the center of the Jog Switch |
| 7 | Quick Color Change | press and hold |  |  |
| 8 | Setting Color Mode | press simultaneously |  |  |
| 9 | Use Markers | short press of |  |  |
| 10 | Adjust Marker Position | press |  |  |
| 11 | Change Contrast | short press of |  |  |
| 12 | Toggle Focus Lock | press and hold |  |  |
| 13 | Check Reading Position | press the simultaneously |  |  |
| 14 | Adjust Volume | press |  |  |
| 15 | Adjust Voice Speed | press the or |  |  |
| 16 | Freeze Image | short press of |  |  |
| 17 | Save Frozen Image | press and hold |  |  |
| 18 | Execute OCR | short press of |  |  |
| 19 | Save OCR | press and hold the OCR button |  |  |
| 20 | Media File Explorer | press and simultaneously |  |  |
| 21 | OCR file Explorer | press and simultaneously |  |  |
| 22 | Document File Explorer | press and simultaneously. |  |  |
| 23 | Run the File Explorer | short press of |  |  |
| 24 | Check Date / Time | press and simultaneously |  |  |
| 25 | Activate Digital Photo Frame | press and simultaneously. |  |  |
| 26 | Start and Stop Screen Saver | press , and simultaneously. |  |  |
| 27 | Reset | press the Power button while holding the center of the Jog Switch |  |  |

Note

The below shortcuts are only available when using the live camera modes.

Warranty Statement

HIMS warrants all GoVision models against defects in materials and workmanship under normal use for a period of TWO years from the date of original purchase when purchased new. Normal wear and tear is exempted from this warranty. HIMS will repair at no labor charge and, at its sole discretion, will repair or replace any parts during the warranty period. Customer shall be responsible for the shipping charge of the product to HIMS or a HIMS authorized service center for repair. Customer may be required to provide “proof of purchase”, usually in the form of the original receipt or invoice, to document the original date of purchase.

This warranty does not cover: 1. Defects or damage resulting from misuse, accident or negligent use, or use in other than a normal or customary manner, 2. Defects or damage resulting from improper installation, operation, maintenance, alterations, adjustments, or modifications, 3. Defects or damage caused by dropping, scratches, spills, liquids, or other substances or misuse to any plastic or other external surfaces, 4. Damage to the product caused by operation with any product not supplied by HIMS.

There are no other expressed or implied warranties applicable to this product. HIMS’ liability under this warranty shall be limited to repair or replacement as described above.  In no event shall HIMS be liable for any special, consequential, or incidental damages.

This warranty is valid only for repair within North America.  This warranty does not apply if the product is used outside North America.

For Technical Support and Service, Contact HIMS Inc.

Phone: 512-837-2000  |   Fax: 512-837-2011
Email: support@hims-inc.com

North American headquarters located at:
4616 W Howard Ln., Ste 960 Austin, TX 78728

- To learn more about HIMS products 
for people with low vision or blindness, 
visit our website: www.hims-inc.com